CYPHORAGRID
Enterprise Network Security, Segmentation & Attack Simulation Lab

Documentation — Stages 10.0 to 10.4
Simple, human-readable project documentation

# 10.0 — Documentation & Professional Presentation

This section explains the completed CyphoraGrid project in a simple and professional way. It covers the network design, IP addressing, security controls, attack testing, incident response, and resilience testing.

# 10.1 — Final Network Architecture

CyphoraGrid uses a Cisco 2911 router, one core switch and two access switches. The router performs inter-VLAN routing. The switches separate users and security zones using VLANs. Redundant links are present between the switches, and STP prevents Layer-2 loops.

R1-EDGE — Cisco 2911 router used for inter-VLAN routing, ACL filtering, SSH management and Syslog forwarding.

SW1-CORE — Cisco 3560 core switch connected to the router, both access switches and the internal servers.

SW2-ACCESS — Cisco 2960 used for Management, Finance and HR endpoints.

SW3-ACCESS — Cisco 2960 used for IT Security and DMZ endpoints.

SW1, SW2 and SW3 have redundant Layer-2 connections.

802.1Q trunking carries the required VLANs between network devices.

Simple topology:

R1-EDGE
   │
SW1-CORE
  /     \
SW2     SW3
  \     /
   ─────

The SW1–SW2–SW3 links create redundancy. STP blocks a redundant path when needed and can use an alternate path when a link fails.

# 10.2 — Final IP Address & VLAN Documentation

VLAN table:

| VLAN | Name | Network | Gateway | Purpose |
| --- | --- | --- | --- | --- |
| 10 | MANAGEMENT | 192.168.10.0/24 | 192.168.10.1 | Management users |
| 20 | FINANCE | 192.168.20.0/24 | 192.168.20.1 | Finance |
| 30 | HR | 192.168.30.0/24 | 192.168.30.1 | HR |
| 40 | IT_SECURITY | 192.168.40.0/24 | 192.168.40.1 | IT/Security |
| 50 | SERVERS | 192.168.50.0/24 | 192.168.50.1 | Internal servers |
| 60 | DMZ | 192.168.60.0/24 | 192.168.60.1 | DMZ and attack simulation |
| 99 | NATIVE_MGMT | 192.168.99.0/24 | 192.168.99.1 | Network management/native VLAN |
| 999 | BLACKHOLE | — | — | Unused-port isolation |

Device and endpoint table:

| Device | IP | VLAN | Gateway | Role |
| --- | --- | --- | --- | --- |
| SW1-CORE | 192.168.99.11 | 99 | 192.168.99.1 | Core management |
| SW2-ACCESS | 192.168.99.12 | 99 | 192.168.99.1 | Access management |
| SW3-ACCESS | 192.168.99.13 | 99 | 192.168.99.1 | Access management |
| PC-MGMT-01 | 192.168.10.10 | 10 | 192.168.10.1 | Management |
| PC-MGMT-02 | 192.168.10.11 | 10 | 192.168.10.1 | Management |
| PC-FIN-01 | 192.168.20.10 | 20 | 192.168.20.1 | Finance |
| PC-FIN-02 | 192.168.20.11 | 20 | 192.168.20.1 | Finance |
| PC-HR-01 | 192.168.30.10 | 30 | 192.168.30.1 | HR |
| PC-HR-02 | 192.168.30.11 | 30 | 192.168.30.1 | HR |
| PC-IT-01 | 192.168.40.10 | 40 | 192.168.40.1 | IT Security |
| PC-IT-02 | 192.168.40.11 | 40 | 192.168.40.1 | IT Security |
| SRV1 | 192.168.50.10 | 50 | 192.168.50.1 | Internal server |
| SRV2 | 192.168.50.11 | 50 | 192.168.50.1 | Syslog server |
| DMZ-SRV1 | 192.168.60.10 | 60 | 192.168.60.1 | DMZ server |
| PC-ATTACKER | 192.168.60.20 | 60 | 192.168.60.1 | Attack simulation |

VLAN 999 is used only for unused switch ports and has no IP subnet. VLAN 99 is the native VLAN on trunks and also carries switch management IPs.

# 10.3 — Security Control Matrix

| Control | Implementation | Why it is used |
| --- | --- | --- |
| VLAN segmentation | VLANs 10/20/30/40/50/60 | Separates departments, servers and DMZ |
| DMZ isolation | DMZ-IN ACL | Blocks DMZ traffic toward protected networks |
| Server protection | SERVER-OUT ACL | Controls access to internal servers |
| SSH security | SSHv2 + local account | Secure device administration |
| Management ACL | MGMT-SSH | Only Management and IT Security networks can SSH |
| Port Security | Maximum 1 MAC | Limits unauthorized devices |
| Sticky MAC | Enabled on protected ports | Learns the expected device MAC |
| Violation mode | Restrict | Restricts violating traffic |
| PortFast | Endpoint ports | Faster endpoint convergence |
| BPDU Guard | Endpoint ports | Protects edge ports from rogue BPDUs |
| Unused-port lockdown | Shutdown + VLAN 999 | Reduces unauthorized physical access |
| Native VLAN | VLAN 99 | Dedicated native/management VLAN |
| STP | PVST | Prevents Layer-2 loops and provides redundancy |
| Syslog | SRV2 192.168.50.11 | Centralizes event transport |
| Incident response | Fa0/10 shutdown/recovery | Demonstrates containment and recovery |

# 10.4 — Attack Simulation & Incident Response Evidence

## 10.4.1 — DMZ to Internal Network Isolation

PC-ATTACKER (192.168.60.20) was used to test whether a DMZ device could reach protected internal networks. The attempts were blocked by the DMZ-IN ACL.

| Target | Result | Control |
| --- | --- | --- |
| 192.168.10.1 — Management | Blocked | DMZ-IN |
| 192.168.20.1 — Finance | Blocked | DMZ-IN |
| 192.168.30.1 — HR | Blocked | DMZ-IN |
| 192.168.40.1 — IT Security | Blocked | DMZ-IN |
| 192.168.50.1 — Servers | Blocked | DMZ-IN |
| 192.168.99.1 — Native Management | Blocked | DMZ-IN |

## 10.4.2 — DMZ Server Access

The attacker could reach DMZ-SRV1 (192.168.60.10). Ping, HTTP and FTP were verified. SSH was refused. This shows that selected DMZ services can be exposed without giving the attacker access to the internal server network.

## 10.4.3 — Lateral Movement Test

PC-ATTACKER attempted to reach SRV1 (192.168.50.10). The connection was blocked, showing that the attacker could not move directly into the internal server VLAN.

## 10.4.4 — Unauthorized SSH Test

SSH management was restricted with the MGMT-SSH ACL. Management and IT Security networks were authorized, while the DMZ attacker was denied access to network-device management.

## 10.4.5 — Port Security Validation

Protected endpoint ports were configured with a maximum of one MAC address, sticky MAC learning and Restrict violation mode. The tested protected ports showed zero security violations.

## 10.4.6 — Incident Response Simulation

1. Detection — Blocked traffic from PC-ATTACKER toward protected networks was observed through ACL activity.

2. Analysis — The source was identified as PC-ATTACKER, 192.168.60.20, in VLAN 60.

3. Containment — SW3 Fa0/10 was shut down to isolate the attacker.

4. Recovery — Fa0/10 was restored using no shutdown.

5. Validation — DMZ connectivity returned, while access from the attacker toward the internal server remained blocked.

## 10.4.7 — Resilience / Link-Failure Test

SW3 Gi0/1 was intentionally shut down to simulate a link failure. The first test exposed that SW2 did not have VLANs 40, 50 and 60 in its VLAN database even though the trunks were configured to allow them. The required VLAN definitions were added to SW2 and verified.

After the correction, PC-IT-01 successfully reached SRV1 through the alternate path with 4/4 replies and 0% packet loss. SW3 Gi0/1 was then restored.

## 10.4.8 — Final Test Summary

| Test | Result |
| --- | --- |
| DMZ → Management | Blocked |
| DMZ → Finance | Blocked |
| DMZ → HR | Blocked |
| DMZ → IT Security | Blocked |
| DMZ → Internal Servers | Blocked |
| DMZ → DMZ Server | Allowed |
| HTTP to DMZ Server | Allowed |
| FTP to DMZ Server | Allowed |
| SSH from authorized network | Allowed |
| SSH from attacker | Blocked |
| Attacker → Internal Server | Blocked |
| Port Security | Verified |
| Unused Ports | Disabled |
| STP Redundancy | Tested |
| Link Failure Recovery | Verified |
| Incident Containment | Tested |
| Incident Recovery | Tested |
| Syslog Transport | Verified |

# Simple Viva Explanation

CyphoraGrid is an enterprise-style network security lab built in Cisco Packet Tracer. I divided the network into different VLANs for departments, servers and the DMZ. The router controls communication between VLANs using ACLs, while the switches use port security, BPDU Guard, PortFast and unused-port shutdown. I also tested attack isolation, SSH security, incident response, Syslog transport and link-failure recovery.

Stages 10.0–10.4 Complete